The current Realtek WiFi USB Linux driver had supported lots more than 10 vip/pid USB WiFi devices. However, some customers want to add new vid/pid(s) into the Realtek WiFi USB Linux driver by themselves. This document will describe the way to let the driver to support the devices with new vid/pid.

The line 59 of os_dep\linux\usb_intf.c file describes all the vid/pids which the Realtek WiFi USB Linux driver supports currently. For the new vid/pid(s) which want to be supported, they can be added into the USB id table named “rtw_usb_id_tbl” array variable.

Ex:

static struct usb_device_id rtw_usb_id_tbl[] ={

//92DU

// Realtek */

{USB_DEVICE(USB_VENDER_ID_REALTEK, 0x8193)},

{USB_DEVICE(USB_VENDER_ID_REALTEK, 0x8194)},

{}   <- the new vid/pid can be added here

};

After adding the new vid/pid into the rtw_usb_id_tbl table, the driver has to be re-compiled. The new driver image will be created and it can support the device with the new vid/pid.